Mission 11:

Spirit Level

Student Workbook

Let’s level up!

In this project you’ll build a spirit level! This is more than just a fun project – it’s a useful tool with practical applications.

Go to the Mission 11 Log and fill out the 
Pre-Mission preparation.

- Your cell phone can detect if it is level or tilted. What other devices use some kind of sensor to determine their orientation?

Mission 11: Spirit Level

How level is your desk or table?

Write some code to find out. During this project you will build a spirit level.

Project Goals:

- Display a numeric "tilt" value from the accelerometer.

- Scale the raw tilt value to show 0° to 90° incline.

- Replace the number display with a graphical ball simulation!

Mission 11: Get started

- Go to https://make.firialabs.com/ and log in.

- Go to Mission 11

- Click  and start Mission 11.

Objective #1: Accel

This mission will use CodeX’s built-in accelerometer

- What is an accelerometer?

- Click on

- Go to the Mission Log and answer the question.

- Return to the digital instructions.

- Go back to the Mission Log and answer the question.

Objective #1: Accel

DO THIS:

- Close the instruction panel

- Use the camera controls to zoom in

- Click on the accelerometer

- Create a new file named Spirit_Level

Objective #2: Tilt-o-Matic

When you read the accelerometer, it returns three values, one for each axis.

- The three values are enclosed in parenthesis: 
(x, y, z)

- This type of data is called 
a tuple.

- Other tuples:

  - Color values:  (red, green, blue)

  - Location on a grid:  (x, y)

To read the accelerometer, use this code:

val = accel.read()

A tuple is a special kind of list. You can get 
specific x, y or z values like items in a list:

tilt_x = val[0]  # first value is at index 0
	tilt_y = val[1] 
	tilt_z = val[2]

Objective #2: Tilt-o-Matic

This mission will use CodeX’s built-in accelerometer

- Go to the Mission Log and answer the questions about data from the accelerometer.

DO THIS:

- Read the accelerometer

- Display the x value

- Delay a short time to read the display

- Your code could look like this:

Objective #3:  Scale to Degrees

- The numbers you see on the display are all over the place.

- We usually measure angles in degrees.

- You need to convert the raw data -- numbers you see on the display -- into degrees.

- This involves some complicated math -- trigonometry -- so use the code given and don’t worry about the actual math.

Objective #3:  Scale to Degrees

DO THIS:

- Import the math module

- Add the code as shown

- Change the display.print to show degrees

- Run the code

  - Can you get CodeX level, 
with 0 degrees?

- Go to the Mission Log and answer the question

Objective #4: Static Ball

Time to learn a little about drawing on the display

Here are the drawing functions you will use for your spirit level:

Concept: The display

The CodeX LCD display is 
240 pixels x 240 pixels

- Each tiny pixel works JUST like the 4 RGB LED pixels at the top of the CodeX.

- x in the (x, y) is the display width

- y is the display height

Objective #4: Static Ball

DO THIS:

- Define a value for the center of the display

- Type the code to draw a line in the center

- Change the display.print() to drawing a circle instead

Objective #5:  Rolling Stone

Time to make that ball move

- Use the information you have:

  - Center of display

  - Degrees for x

- Use math to calculate the position of the circle

- Use a variable for the data

Objective #5:  Let’s Keep Playing

DO THIS:

- Define a value for the x position

- Assign it the value:

  - x = CENTER + degrees

- Use the variable in the draw_circle() command

- Run the code and tilt CodeX to see the ball move

Mission Quiz: Accelisplay

Test your skills by taking the quiz.

Objective #6: Eraser First

Spirit Level -- final touches

- Why is the ball drawing on top of itself?

- Because you are not erasing it first.

- Cover your tracks!

- Draw a white circle to “erase” the current circle before moving to a new position and drawing a new orange circle.

Objective #6:  Reduce Repetition

DO THIS:

- Define the x variable above the while loop

- Draw a white circle before calculating a 
new x value

- Run your program 
Does it work correctly?

Mission Complete

You have completed the eleventh mission.

Do this:

- Read your “Completed Mission” message

- Complete your Mission 11 Log

  - Post-Mission Reflection

- Get ready for your next mission!

Wait! Before you go … Clear the CodeX

Go to FILE -- BROWSE FILES

Select the “Clear” file and open it

Run the program to clear the CodeX

Okay. Now you can go.